| СПЕЦИФИКАЦИЯ НА ПРОДУКТ Наименование продукта; Краситель пищевой ### «Сахарный колер IV” Е150d. Коммерческое название; Краситель пищевой ### «Карамельный колер». Тип; -. ОРГАНОЛЕПТИЧЕСКИЕ ПОКАЗАТЕЛИ. Внешний вид; Вязкая жидкость. Запах; Характерный для данного наименования, без посторонних запахов. # Цвет; Темно-коричневый. ФИЗИКО-ХИМИЧЕСКИЕ ПОКАЗАТЕЛИ. Растворимость; Растворим в воде. Плотность при 20°С, г/см³; 1,24-1,28. Поглощение света при 610 нм 0,1% раствора 10 мм кювета; 0,245-0,27. Красящая способность ЕВС; 48.000-75.000. Брикс при 20°С, %; 50-60. рН; 2,4-3,5. Общее содержание серы (S), %; 1,4-10. Содержание диоксида серы (SO2), %, не более; 0,2. Общее содержание азота (N), %; 0,5-3,5. Аммиачный азот %, не более; 0,6. 4-метилимидазол, мг/кг, не более; 250*. Посторонние примеси; Не допускаются. #### Показатели безопасности. #### ТОКСИЧНЫЕ ЭЛЕМЕНТЫ (В соответствии с ТР ТС 029/2012 Приложение 28). Свинец, мг/кг, не более; 2,0. Мышьяк, мг/кг, не более; 1,0. Кадмий, мг/кг, не более; 1,0. Ртуть, мг/кг, не более; 1,0. #### УПАКОВКА, ХРАНЕНИЕ. Упаковка Пластиковые канистры с закручивающимися крышками или другие виды упаковки, обеспечивающие сохранность продукта при его транспортировке и хранении. Маркировка Обязательная с указанием: - наименования продукта, - наименования и местонахождения (адрес, включая страну) изготовителя, - состава продукта (кроме продукции, состоящей из одного компонента), - даты изготовления, - срока годности, - номера партии, - массы нетто, массы брутто (или массы тароупаковки), - условий хранения. Примечание: маркировка должна быть также на русском языке. Срок годности; Не менее 12 месяцев с даты производства, в закрытой оригинальной упаковке при рекомендованных условиях хранения.. Продукт не должен содержать генетически модифицированные источники пищи.. Примечание: Груз должен поступать с сопроводительными документами, а именно: - свидетельством о государственной регистрации / декларацией о соответствии, - удостоверением качества и безопасности / сертификатом анализа и спецификацией на продукт. Остаточный срок годности товара при поставке должен составлять не менее 2/3 от установленного на данный вид. | PRODUCT SPECIFICATION Product name; ### Food Colour ### «Caramel color IV» Е150 d. Trade name; ### Food Colour ### «Caramel Сolor». Type; ### -. ORGANOLEPTIC CHARACTERISTICS. Appearance; Viscous liquid. Odor; Typical for this product, without foreign odors. # Color; Dark brown. PHYSICAL-CHEMICAL PARAMETERS. Solubility; Soluble in water. Density at t 20°С, g/ml; 1,24-1,28. Light absorption at 610 nm 0.1% solution, 10 mm cuvette; 0,245-0,27. Color, E.V.S. units; 48.000-75.000. Brix, at t 20°С, %; 50-60. pH; 2,4-3,5. Total sulfur (as S), %; 1,4-10. Sulfur dioxide, (as SO2), %, not more than; 0,2. Total nitrogen(as N), %; 0,5-3,5. Ammoniacal nitrogen%, not more than; 0,6. 4-Metthylimidazole (MEI), mg/kg, not more than; 250*. Foreign impurities; Not allowed. Product safety parameters. TOXIC DATA. Lead, mg / kg, not more; 2,0. Arsenic, mg / kg, not more; 1,0. Cadmium, mg / kg, not more; 1,0. Mercury, mg / kg, not more; 1,0. #### PACKAGE, STORAGE. Package Plastic food cans with screw caps or other types of packaging that ensure the safety of the product during its transportation and storage. Marking Mandatory stating:  name of the product,  name and location (address, including country) of the manufacturer,  content of the product (except for products consisting of one component),  date of manufacture,  expire date,  batch number,  net weight, gross weight (or mass of package)  storage conditions. Note: The marking should also be in Russian. Shelf life; Not less than 12 months from the date of production, in unopened original packaging under recommended storage conditions.. The product must not contain genetically modified food sources.. Note: Products should be accompanied with the following documents: - certificate of state registration / declaration of conformity, - certificate of quality and safety / certificate of analysis and specification of the product. Remaining shelf life of the products when delivered should be at least 2/3 of the established for this product type. |
| --- | --- |
| * для красителя, стандартизованного на интенсивность окрашивания 0,1 | * calculated on an equivalent colour basis |